OPENEYE OE-C2212B4 A&E SPECIFICATIONS
4MP Outdoor IP Bullet Camera
www.openeye.net

DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

Section 28 00 00 Electronic Safety and Security

Section 28 20 00 Video Surveillance

Section 28 21 00 Surveillance Cameras

Section 28 23 00 Video Management System

PART 1 GENERAL

  - SUMMARY

- This section includes the minimum requirements for Security Cameras as a Service platform comprised of, but not limited to, the following components:

  - Network Camera Hardware

  - Network Camera Firmware

  - Network Video Management Software

  - Online Web Services

- References

  - 28 05 00 Common Work Results for Electronic Safety and Security

  - 28 05 25 Cloud Based Storage for Electronic Safety and Security

  - 28 21 13.11 IP Cameras

  - 28 21 13.13 Analytic Packages for IP Cameras

  - SUBMITTALS

- Manufacturer’s data sheets in digital or printed form.

- Manufacturer’s installation and operations manuals in digital or printed form.

  - QUALIFICATIONS

- Manufacturer

  - Manufacturer shall have been in business for more than 10 years.

- Installer

  - All installation, configuration, setup and related work shall be performed by an authorized technician certified by the manufacturer.

  - All installation shall be performed by a technician licensed to install and service video surveillance and security equipment as mandated by the authority having jurisdiction.

  - WARRANTY

- The manufacturer shall warrant camera for a period of 5 years for the repair or replacement of defective equipment.

- END OF SECTION -

- SECTION 2 PRODUCTS

DOCUMENT SUMMARY

- This document specifies the minimum criteria for the design and performance of the OE-C2212B4 Network Video Camera.

STANDARDS

- The Network Video Camera meets or exceeds the following requirements of the following publications and organizations.

  - Joint Photographic Experts Group (JPEG)

  - Moving Picture Experts Group (MPEG) (H.264/MPEG-4 Part 10 AVC)

- The Network Video Camera meets or exceeds the following mechanical standards:

  - IP67 Ingress Protection

SYSTEM DESCRIPTION

- The Network Video Camera shall offer the latest in digital capture technology, product reliability, security, ease of use, and customer support structure.

- The Network Video Camera shall be an outdoor camera. The Network Video Camera shall come equipped with an IR Cut Filter for day and night video.

OPERATING SPECIFICATIONS

- Environmental Conditions: the Network Video Camera shall be designed to operate under the following environmental specifications:

  - Operating Temperature: -22°F~140°F (-30°C~60°C) non-condensing.

- Power Requirements: Components shall have the following electrical specifications:

  - Power Requirement: PoE (IEEE 802.3af)/12vDC±25%

  - Max power: 9.6 Watts

  - Class 3 PoE will power all available functions.

- Product Label Requirements:

  - The Network Video Camera shall have the FCC and UL certifications clearly marked on the product and box label.

    - Dimensions:

      - Package: 9.2” x 4.4” x 3.9” (233 x 113 x 98mm)

      - Package Label: 2.5” x 1.9” (63 x 50mm)

      - Product Label: 1” x 0.8” (26 x 20mm)

    - Weight: 1.12lbs (.51kg)

OPERATIONAL REQUIREMENTS

- NETWORK VIDEO CAMERA – The network video camera shall include, as a minimum, the following features/functions/specifications. The network video camera shall:

  - Include a built-in memory card slot, available for microSD/SDHC/SDXC memory card, capacity up to 512GB.

  - Include a high-quality Progressive CMOS image sensor.

  - Include a varifocal 2.8~12mm Motorized Zoom Lens.

  - Have a field of view of 86° (wide)~36° (tele).

  - Include wide dynamic range (WDR) function balancing situations with dark lighting conditions in some areas.

  - Operate with a minimum illumination of 0.003 lux in day mode (F1.6, AGC ON) and 0 lux in night mode (IR on).

  - Include integrated IR with a range of up to 164’.

  - Have a white balance function that is adjustable through the camera’s web interface.

  - Have a backlight compensation function that is adjustable through the camera’s web interface.

  - Include a mechanical IR cut filter that is removed during low light conditions to improve low light imaging. This shall be referred to as a True Day/Night function.

  - Have an auto gain control function that is adjustable through the camera’s web interface.

  - Include 1 built-in microphone.

  - Have a reset button on the camera to restore the device to default parameters.

- TRANSMISSION CAPABILITIES – The network video camera shall provide the following characteristics/features/functions. The network video camera capabilities shall:

  - Be capable of encoding video for network transmission across a LAN or WAN.

  - Be capable of compressing and transmitting video data using MJPEG and H.264 compression methods.

  - Be compliant with the ONVIF standard for Network Video Transmitters (NVT).

  - Have user authentication for secure video transmission and shall be capable of up to 32 user accounts.

  - Be capable of providing up to two individually configured H.264 video profiles.

  - Be capable of providing video in MJPEG and H.264 at resolutions as high as 2688 x 1520, or 4MP.

  - Be capable of providing video at the following resolutions: 2688 x 1520, 2560 x 1440, 2304 x 1296, 1920 x 1080, 1280 x 720, 720 x 576, 640 x 360.

  - Provide 4MP video at a maximum rate of 30 Images per Second (IPS) when set to H.264.

  - Provide 4MP video at a maximum rate of 5 Images per Second (IPS) when set to MJPEG.

  - Allow the user to set the Group of Pictures (GOP) rate for the H.264 profiles to fine tune the video transmission.

  - Include framerate control that allows the user to enter any rate from 0-30 (dependent on resolution).

  - Include a video compression setting that allows the user to set the video compression bit rate.

  - Include the ability to switch from Variable Bit Rate (VBR) mode to Constant Bit Rate (CBR) mode.

  - Include the ability to enable Smart Encoding to allow the user to dynamically reduce their throughput.

  - Include the following Image Rotation options:

    - Normal

    - Flip Vertical

    - Flip Horizontal

    - 180°

    - 90° Clockwise

    - 90° Counter-clockwise

  - Include a Motion Detection function.

    - The Motion Detection function must allow for 4 separate motion detection areas.

    - The Motion Detection function must allow for configurable sensitivity and object detection size.

    - The Motion Detection function must have configurable actions that occur when motion is detected, including sending a notification email with a JPG attachment, triggering the alarm output on the camera, recording the video stream for a pre-defined duration to an internal microSD card, saving pre-defined number of snapshot images to an internal microSD card.

  - Have the ability to sync with an NTP server.

  - Have the ability to be discovered over multiple sub-nets with the use of a Camera Discovery Tool.

  - With the Camera Discovery tool have the ability to set IP fields, camera settings, and program firmware on multiple cameras at the same time.

- KEY ACCESSORIES – The camera shall have the following accessories for mounting and controlling the camera in various settings;

  - OE-CA00JB-01 – Junction Box to add conduit connection.

  - OE-CA00PMK-01 – Pole Mount Kit

- LIST OF SUPPORTED MODELS

  - The Network Video Camera shall be the following model or an approved equivalent:

| OE-C2212B4 | 4MP Outdoor IP Bullet Camera with varifocal 2.8~12mm Motorized AF lens |
| --- | --- |

PART 3 EXECUTION

  - INSTALLATION

- Installer shall comply with all instructions and best practices specified by the hardware and software manufacturer.

- Installer shall comply with all applicable state and local regulatory requirements.

  - STORAGE

- Equipment shall be stored in environmental conditions within the stated temperature and humidity ranges specified by the hardware manufacturer.

  - COMISSIONING

- The manufacturer shall offer the option for factory pre-configuration of all hardware and software purchased from the manufacturer as an additional paid service.

- The manufacturer shall offer the option for remote or in person system commissioning as an additional paid service.

- END OF SECTION -

END OF DOCUMENT