OPENEYE OE-C8213 A&E SPECIFICATIONS
3MP IP PTZ Dome
WWW.OPENEYE.NET

DIVISION 28 - ELECTRONIC SAFETY AND SECURITY

Section 28 00 00 Electronic Safety and Security

Section 28 20 00 Video Surveillance

Section 28 05 19 Storage Appliances for Electronic Safety and Security

Section 28 23 00 Video Management System

PART 1 GENERAL

  - SUMMARY

- This section includes the minimum requirements for Security Cameras as a Service platform comprised of, but not limited to, the following components:

  - Network Camera Hardware

  - Network Camera Firmware

  - Network Video Management Software

  - Online Web Services

- Related Requirements

  - 28 05 00 Common Work Results for Electronic Safety and Security

  - 28 05 25 Cloud Based Storage for Electronic Safety and Security

  - 28 21 13.11 IP Cameras

  - 28 21 13.13 Analytic Packages for IP Cameras

  - SUBMITTALS

- Manufacturer’s data sheets in digital or printed form.

- Manufacturer’s installation and operations manuals in digital or printed form.

  - QUALIFICATIONS

- Manufacturer

  - Manufacturer shall have been in business for more than 10 years.

- Installer

  - All installation, configuration, setup and related work shall be performed by an authorized technician certified by the manufacturer.

  - All installation shall be performed by a technician licensed to install and service video surveillance and security equipment as mandated by the authority having jurisdiction.

  - WARRANTY & SUPPORT

- The Network Video Camera shall come with a minimum 2-year manufacturer’s warranty with the 1st year including advance replacement service.

- The Network Video Camera shall come with a heroic customer service support structure, including a 1-888 number for technical service, pre and post sales assistance, and full access to online technical support as well as online sales support.

- Technical support shall be available weekdays from 5 a.m. to 5 p.m. PST.

- Firmware updates and patches shall be available free of charge.

- END OF SECTION -

- SECTION 2 PRODUCTS

DOCUMENT SUMMARY

- This document specifies the minimum criteria for the design and performance of the OE-C8213 Network Video Camera.

STANDARDS

- The Network Video Camera meets or exceeds the following requirements of the following publications and organizations.

  - Joint Photographic Experts Group (JPEG)

  - Moving Picture Experts Group (MPEG) (H.264/MPEG-4 Part 10 AVC)

SYSTEM DESCRIPTION

- The Network Video Camera shall offer the latest in digital capture technology, product reliability, security, ease of use, and customer support structure.

- The Network Video Camera shall be an indoor/outdoor camera with an IP66 rated housing to protect against weather conditions. The Network Video Camera shall have an integrated heater for extreme temperatures as low as -40°F (-40°C). The Network Video Camera shall come equipped with an IR Cut Filter for day and night video.

OPERATING SPECIFICATIONS

- Environmental Conditions: the Network Video Camera shall be designed to operate under the following environmental specifications:

  - Operating Temperature: -40°F to 122°F (-40°C to 50°C) non-condensing.

- Power Requirements: Components shall have the following electrical specifications:

- A.	Power Requirement: 820.3at PoE+ / 24 VAC

- Max power: 25.4W

- Rated Amperage: 529.17mA @ PoE+ / 1058.33mA @ 24VAC

- Product Label Requirements:

  - The Network Video Camera shall have the FCC and UL certifications clearly marked on the product and box label.

    - Dimensions: 8.2in (210mm) H: 11.1in (279mm)

    - Weight: 6.39 lbs (2.9 kg)

OPERATIONAL REQUIREMENTS

- NETWORK VIDEO CAMERA – The network video camera shall include, as a minimum, the following features/functions/specifications. The network video camera shall:

  - Include a high quality Sony Progressive CMOS image sensor.

  - Have an ingress protection (IP) rating equivalent to IP66 against the ingress of dust and moisture.

  - Include digital wide dynamic range (D-WDR) function balancing situations with dark lighting conditions in some areas.

  - Operate with a minimum illumination of 0.04 lux in day mode and 0.002 lux in night mode at 50IRE.

  - Include a mechanical IR cut filter that is removed during low light conditions to improve low light imaging. This shall be referred to as a True Day/Night function.

  - Include a motorized lens with a length from 4.3mm to 170mm for a total optical zoom of 30x.

  - Have a field of view from 62° at the widest view to 2° at the narrowest view.

  - Have Auto/Manual Iris control that is adjustable through the camera’s web interface.

  - Have Auto/Manual white balance function that is adjustable through the camera’s web interface.

  - Have a white balance range of at least 2700K to 7800K.

  - Have a backlight compensation function that is adjustable through the camera’s web interface.

  - Have an auto gain control function that is adjustable through the camera’s web interface.

  - Include 4 alarm inputs used for activating alarm events.

  - Include 2 relay outputs that can be activated on alarm input and on motion detection.

- PTZ FUNCTIONS – The network video camera shall provide the following characteristics/features/functions. The network video camera shall:

  - Have an optical zoom equal to 30x.

  - Have a pan range that is an endless 360 degrees.

  - Have a tilt range of -10° to 100°.

  - Support up to 256 preset positions.

  - Have a preset accuracy within no less than ±0.225°.

  - Have a programmable move speed that shall be 5° per second minimum and 400° per second maximum.

  - Be capable of storing and recalling 8 pattern (mimic) tours.

  - Be capable of storing and recalling 8 Tours consisting of stored preset positions.

  - Be capable of storing and recalling 4 “Auto-scan” tours, consisting of two preset positions that the camera shall scan between.

  - Support 20 “Privacy Mask” areas.

  - Have a picture function that shall include an image flip function that can be turned on and off through the camera’s web interface.

  - Have a home setting that the camera shall return to after a user defined amount of time that shall be a minimum of 1 minute and a maximum of 128 minutes. When the auto-home setting is activated, it shall call a Preset Tour, a Pattern Tour, an Auto-scan Tour, or a Preset position.

- TRANSMISSION CAPABILITIES – The network video camera shall provide the following characteristics/features/functions. The network video camera capabilities shall:

  - Be capable of encoding video for network transmission across a LAN or WAN.

  - Be capable of compressing and transmitting video data using both MJPEG and H.264 compression methods.

  - Be compliant with the ONVIF standard for Network Video Transmitters (NVT).

  - Have user authentication for secure video transmission, and shall be capable of up to 20 user accounts.

  - Be capable of providing up to four individually configured H.264 video streams, or three H.264 and one MJPEG video streams.

  - Be capable of providing video in MJPEG and H.264 at resolutions as high as 2048 x 1536, or 3MP.

  - Be capable of providing video at the following resolutions: 2048x1536, 1920 x 1080, 1280 x 1024, 1280 x 720, 1024 x 768, 800 x 600, 720 x 480, 640 x 480, 352 x 240.

  - Provide 3MP video at a maximum rate of 60 Images per Second (IPS) when set to H.264.

  - Provide 2MP video at a maximum rate of 30 Images per Second (IPS) when set to MJPEG.

  - Allow the user to set the Group of Pictures (GOP) rate for the H.264 streams to fine tune the video transmission.

  - Allow the user to set the H.264 profile as H.264 baseline profile, H.264 main profile, or H.264 high profile to control how the camera encodes video.

  - Include framerate control that allows the user to enter any rate from 0-60 (dependent on resolution).

  - Include a video compression setting that allows the user to set the video compression bit rate.

  - Include the ability to choose from Variable Bit Rate (VBR) mode, Constant Bit Rate (CBR) mode, or Low bit rate.

  - Include a Motion Detection function.

    - The Motion Detection function must allow for 10 separate motion detection areas.

    - The Motion Detection function must allow for configurable sensitivity and motion detection event durations

    - The Motion Detection function must have configurable actions that occur when motion is detected, including ;

      - Sending alarm message and/or image by email

      - Sending alarm message and/or image by FTP

      - Triggering the alarm output on the camera

      - Recording the video stream for a pre-defined duration to an internal microSD card.

  - Have the ability to sync with an NTP server.

  - Have the ability to be discovered over multiple sub-nets with the use of a Camera Discovery Tool.

    - With the Camera Discovery tool have the ability to set IP fields, camera settings, and program firmware on multiple cameras at the same time.

- KEY ACCESSORIES – The camera shall have the following accessories for mounting and controlling the camera in various settings.

  - Threaded Mount Adapter (Included)

  - OE-CA00CMP-01 – Ceiling-Mounted Pole

  - OE-CA00WM-01 – Indoor/Outdoor Wall Mount

  - OE-CA00CM-01 – Corner Mount

  - OE-CA00PMK – Pole Mount Kit

  - CA-501J – 3 Axis Joystick Controller*

*Requires Direct PTZ or Recorder to control

- LIST OF SUPPORTED MODELS

  - The Network Video Camera shall be one of the following models or an approved equivalent:

| OE-C8213 | 3MP Outdoor IP PTZ Camera |
| --- | --- |

PART 3 EXECUTION

  - INSTALLATION

- Installer shall comply with all instructions and best practices specified by the hardware and software manufacturer.

- Installer shall comply with all applicable state and local regulatory requirements.

  - STORAGE

- Equipment shall be stored in environmental conditions within the stated temperature and humidity ranges specified by the hardware manufacturer.

  - COMISSIONING

- The manufacturer shall offer the option for factory pre-configuration of all hardware and software purchased from the manufacturer as an additional paid service.

- The manufacturer shall offer the option for remote or in person system commissioning as an additional paid service.

- END OF SECTION -

END OF DOCUMENT